Task 1: IPv4 vs. IPv6 Addressing Comparison

| Feature / Property | IPv4 Addressing | IPv6 Addressing |
| --- | --- | --- |
| Address Length & Space | 32 bits (4 bytes) yielding approximately 232≈4.3 billion addresses. | 128 bits (16 bytes) yielding approximately 2128≈3.4×1038 addresses. |
| Format & Notation | Dotted-decimal notation divided into 4 octets (e.g., 192.168.1.1). | Hexadecimal notation divided into 8 groups of 4 digits separated by colons (e.g., 2001:0db8:85a3::8a2e:0370:7334). |
| Address Configuration | Relies on manual static assignment or dynamic allocation via DHCP. | Supports DHCPv6 as well as stateless automatic autoconfiguration (SLAAC via NDP/Router Advertisements). |
| Header Complexity & Security | Variable header length (20 to 60 bytes with options); IPsec is optional and relies heavily on NAT to conserve public space. | Fixed-length 40-byte base header for faster routing; designed with built-in end-to-end IPsec support; eliminates the need for NAT. |

Task 2: Static IP Allocation Plan (192.168.10.0/24)

- Network Address: 192.168.10.0

- Subnet Mask: 255.255.255.0 (/24)

- Default Gateway (Router): 192.168.10.1

| Device | Assigned Static IP | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Laptop | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| Smartphone | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 |
| Smart TV | 192.168.10.20 | 255.255.255.0 | 192.168.10.1 |
| Printer | 192.168.10.30 | 255.255.255.0 | 192.168.10.1 |

Task 3: Subnetting Calculations for 192.168.1.0/27

Calculations:

- Base Network: 192.168.1.0 (Class C default mask is /24)

- Borrowed Subnet Bits (n): 27-24=3 bits.

- Number of Valid Subnets: 2n=23=8 subnets.

- Host Bits Remaining (h): 32-27=5 bits.

- Hosts per Subnet (Total): 2h=25=32 addresses.

- Usable Hosts per Subnet: 2h-2=32-2=30 usable hosts.

- Block Size (Increment): 256-32=224 (where mask is 255.255.255.224).

First Three Valid Subnet Ranges:

- Subnet 1:

- Network Address: 192.168.1.0

- Usable Host Range: 192.168.1.1 – 192.168.1.30

- Broadcast Address: 192.168.1.31

- Subnet 2:

- Network Address: 192.168.1.32

- Usable Host Range: 192.168.1.33 – 192.168.1.62

- Broadcast Address: 192.168.1.63

- Subnet 3:

- Network Address: 192.168.1.64

- Usable Host Range: 192.168.1.65 – 192.168.1.94

- Broadcast Address: 192.168.1.95

Task 4: Subnetting Determination for 10.0.5.17 with Mask 255.255.255.0

Step-by-Step Calculation:

- CIDR Notation: 255.255.255.0 corresponds to twenty-four contiguous 1s 				(11111111.11111111.11111111.000000002), which is /24.

- Network Address (Bitwise AND):

- Octet 1: 10 AND 255=10

- Octet 2: 0 AND 255=0

- Octet 3: 5 AND 255=5

- Octet 4: 17 AND 0=0 (000100012 AND 000000002=000000002)

- Network Address: 10.0.5.0

- Broadcast Address:

- Setting the 8 host bits in the 4th octet to all 1s (111111112=255).

- Broadcast Address: 10.0.5.255

- Valid Host Range:

- First Usable Host: Network Address + 1 = 10.0.5.1

- Last Usable Host: Broadcast Address – 1 = 10.0.5.254

Summary of Results:

- Network Address: 10.0.5.0

- Broadcast Address: 10.0.5.255

- Valid Host Address Range: 10.0.5.1 – 10.0.5.254

Task 5: Verification via Subnet Calculator

The subnet calculation was verified using an online subnet calculator with the IP address 10.0.5.17 and subnet mask /24.

Verification: The manual calculation matched the subnet calculator output: Network Address 10.0.5.0, Broadcast Address 10.0.5.255, and usable host range 10.0.5.1–10.0.5.254.