Task 1: Ping Test to [www.youtube.com]

- Command Executed:
	ping www.youtube.com

- Result:
	Pinging www.youtube.com [2001:4860:4826:400::] with 32 bytes of data:
	Reply from 2001:4860:4826:400::: time=61ms
	Reply from 2001:4860:4826:400::: time=53ms
	Reply from 2001:4860:4826:400::: time=54ms
	Reply from 2001:4860:4826:400::: time=56ms

Ping statistics for 2001:4860:4826:400:::
	    Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),
	Approximate round trip times in milli-seconds:
	    Minimum = 53ms, Maximum = 61ms, Average = 56ms

Task 2: Traceroute to [www.flipkart.com] / (https://www.flipkart.com)

- Command Executed:

- Windows: tracert [www.flipkart.com] / (https://www.flipkart.com)

- Linux/macOS: traceroute [www.flipkart.com] / (https://www.flipkart.com)

- Result:

Tracing route to e348826.dsca.akamaiedge.net [64:ff9b::b856:f8b1]
	over a maximum of 30 hops:

1     2 ms     2 ms     1 ms  2409:40c1:4149:f21::47
	  2     *        *        *     Request timed out.
	  3    74 ms    42 ms    58 ms  2405:200:5210:3:3925::ff03
	  4    71 ms    61 ms    58 ms  2405:200:898:3632:62::9
	  5    54 ms    42 ms    48 ms  64:ff9b::c0a8:e3c3
	  6    49 ms    90 ms    45 ms  64:ff9b::c0a8:f48a
	  7     *        *        *     Request timed out.
	  8     *        *        *     Request timed out.
	  9     *        *        *     Request timed out.
	 10    64 ms    47 ms    56 ms  a184-86-248-1.deploy.static.akamaitechnologies.com 	[64:ff9b::b856:f801]
	 11    63 ms    60 ms    48 ms  a184-86-248-177.deploy.static.akamaitechnologies.com 	[64:ff9b::b856:f8b1]

Trace complete.

Task 3: DNS Query with nslookup for [www.zomato.com]

- Command Executed:
	nslookup www.zomato.com

- Result:

Server:  UnKnown
	Address:  10.214.8.246

Non-authoritative answer:
	Name:    e70929.dsca.akamaiedge.net
	Addresses:  2600:140f:7200:49::17d4:65
	                   2600:140f:7200:49::17d4:73
		    184.86.248.194
		    184.86.248.161

Aliases:  www.zomato.com
		 zomato.edgekey.net

Task 4: Local LAN Device Ping Test & Offline Behavior

A friend's laptop has the IP address: 192.168.1.20

- Command Executed:
	ping 192.168.1.20

- When the Device is Online: 
	Reply from 192.168.1.20: bytes=32 time=2ms TTL=128
	Reply from 192.168.1.20: bytes=32 time=2ms TTL=128
	This indicates that the device is reachable on the local network.

- When the Device is Offline
	Request timed out.
	Request timed out.
	Request timed out.
	Request timed out.

If the device is offline, disconnected, blocking ICMP traffic, or unreachable, the computer may receive Request timed out messages.

Result: Ping can be used to check whether another device is reachable on the local network.

Task 5: Diagnostic Summary Report

- (a)Diagnosing "Website Not Loading": ping is useful for checking whether a website or device is reachable, such as using ping www.youtube.com and checking the response time.

- (b) Diagnosing "Slow Connection / Latency Spikes": tracert is useful for diagnosing a slow connection because it shows the hops and response times along the path, such as tracert www.flipkart.com.

- (c) Diagnosing "Suspected DNS Issues": nslookup is useful when a website may have a DNS problem because it checks whether the domain name can be resolved to an IP address, such as nslookup www.zomato.com. If ping works using an IP address but the website name cannot be resolved, DNS may be the problem. Therefore, ping checks connectivity, tracert helps identify delays in the network path, and nslookup checks DNS resolution.