Task 1: Common USB Keyboard and Mouse Problems

| No. | Common Issue | Possible Solution |
| --- | --- | --- |
| 1 | Keyboard or mouse is not detected | Disconnect the device and reconnect it to another USB port. Restart the laptop if necessary. |
| 2 | Cursor lags or moves erratically (mouse) | lean the optical/laser sensor and try a different surface or mousepad. |
| 3 | Keys not registering or double-typing | Clean under the keys with compressed air, and reinstall the keyboard driver from Device Manager. |
| 4 | Wireless keyboard/mouse disconnects randomly | Replace the batteries and move the USB receiver away from other wireless devices (routers, phones) causing interference. |
| 5 | USB port stops working after sleep/wake | In Device Manager, open the USB port's Properties → Power Management tab, and uncheck “Allow the computer to turn off this device to save power.” |

Troubleshooting order

Check USB connection  >  Try another USB port  >  Restart laptop  >  Check Device Manager  >  Update/reinstall driver  >  Test device on another computer

Task 2: Bluetooth Headphones Not Connecting to Phone

Step-by-step troubleshooting

- Check the headphone power

  - Turn the headphones on.

  - Check whether the power/battery indicator is active.

  - Charge the headphones if the battery is low.

- Enable Bluetooth on the phone

  - Open Settings → Bluetooth.

  - Make sure Bluetooth is switched ON.

- Put headphones into pairing mode

  - Press and hold the Bluetooth/power button according to the headphone instructions.

  - Check for a flashing Bluetooth indicator or pairing voice prompt.

- Check the available devices

  - Look under available Bluetooth devices on the phone.

  - Check whether the headphones appear in the list.

- Try connecting

  - Tap the headphone name.

  - Wait until the status changes to Connected.

- Remove an old pairing

  - If the headphones appear under previously connected devices but will not connect, select them and choose Forget/Unpair.

- Pair the headphones again

  - Put the headphones back into pairing mode.

  - Search for Bluetooth devices again.

  - Select the headphones and pair them.

- Check whether they are connected to another device

  - Turn off Bluetooth on nearby laptops, tablets, or other phones.

  - Some headphones automatically reconnect to a previously paired device.

- Restart both devices

  - Turn Bluetooth off and on again.

  - Restart the phone.

  - Turn the headphones off and back on.

- Test the headphones

- Play music or a video.

- Check whether sound is coming through the headphones.

Troubleshooting flow

Headphones not connecting  >  Check battery/power  >  Bluetooth ON on phone?  >  Put headphones in pairing mode  >  Headphones visible?

┌────┴────┐

No     	  	    Yes

↓        	  	      ↓

Restart/   	 Select device

re-pair        	        ↓

Connected?

┌────┴────┐

No       	        Yes

↓       	          ↓

Forget device     Test audio and pair again

Task 3: External Webcam Not Detected in Zoom on Windows 10

Six-point troubleshooting checklist

1. Check the physical connection

- Make sure the webcam USB cable is securely connected.

- Try another USB port.

- If possible, test the webcam on another computer.

2. Check Device Manager

- Right-click Start → Device Manager.

- Look under Cameras, Imaging devices, or Sound, video and game controllers.

- Check whether the webcam appears.

- If there is a warning symbol, update or reinstall the driver.

3. Check Windows camera privacy settings

Go to: Settings → Privacy → Camera

Make sure:

- Allow access to the camera on this device is enabled.

- Allow apps to access your camera is enabled.

- Desktop-app camera access is enabled for applications such as Zoom.

4. Check Zoom's camera settings

Open Zoom and go to: Settings → Video

Check the Camera drop-down menu and select the external webcam instead of the laptop's built-in camera.

5. Close other applications using the webcam

Close applications such as:

- Microsoft Teams

- Skype

- Google Meet

- Camera application

- Other video-conferencing software

Then restart Zoom and test the camera again.

6. Restart/update the system

- Restart the computer.

- Check Windows Update.

- Update the webcam driver if a newer driver is available.

- Reconnect the webcam and test it in the Windows Camera application.

Quick troubleshooting flow

Webcam not detected in Zoom  >  Check USB connection  >  Check Device Manager  >  Check Windows Camera Privacy  >  Check Zoom Camera Settings  >  Close other camera applications  >  Restart / Update driver  >  Test webcam.

Task 4: Printer Showing "Offline" in Windows

When a printer shows Offline, Windows is unable to communicate with the printer normally. The problem may be caused by the printer being powered off, a USB/network connection problem, incorrect printer settings, a driver issue, or a print spooler problem.

Step-by-step troubleshooting using Windows Troubleshooter

Step 1: Check the printer

- Make sure the printer is switched ON.

- Check the printer display for error messages.

- Check that paper and ink/toner are available.

- For a USB printer, check the USB cable.

- For a Wi-Fi printer, confirm that the printer is connected to the correct Wi-Fi network.

Step 2: Open Windows Settings

On Windows 10: Start → Settings → Devices → Printers & scanners

Select the affected printer.

Step 3: Run the troubleshooter

Select: Manage → Run the troubleshooter

Windows will begin checking the printer.

Step 4: Allow Windows to diagnose the problem

The troubleshooter may check:

- Printer connection

- Printer configuration

- Driver problems

- Printer status

- Communication problems

- Print queue/spooler-related issues

- Default printer settings

Step 5: Follow the recommended fix

Windows may identify a problem and recommend an action, such as:

- Setting the printer as the default printer

- Clearing a print queue

- Fixing a connection

- Updating/reinstalling a driver

- Changing printer configuration

Apply the recommended fix when appropriate.

Step 6: Test the printer

After troubleshooting:

- Return to Printers & scanners.

- Check that the printer status is no longer Offline.

- Print a test document.

- Confirm that the document reaches the printer successfully.

Information Provided by the Troubleshooter

During the process, Windows Troubleshooter can provide information about:

- Problem detected: The issue Windows has identified.

- Device status: Whether Windows can communicate with the printer.

- Connection status: Whether the printer is reachable.

- Driver/configuration issues: Whether Windows detects a software or configuration problem.

- Actions taken: Fixes that Windows has automatically applied.

- Problems that could not be fixed: Issues requiring manual troubleshooting.

Printer troubleshooting flow

Printer shows "Offline"  >  Check power and cables  >  Settings  >  Devices  >  Printers & scanners  >  Select printer  >  Manage  >  Run the troubleshooter  >  Windows diagnoses the problem  >  Review detected problem  >  Apply recommended fix  >  Print test page  >  WORKING.