Task 1 — Connect and verify a USB webcam

- Plug the USB webcam into an available USB port on the computer.

- Windows automatically detects the device and installs a generic UVC (USB Video Class) driver — most modern webcams work with this out of the box.

- If a specific driver is required (for extra features like autofocus control or a companion app), download it from the manufacturer's website and run the installer.

- Open Device Manager → Cameras (or Imaging devices) and confirm the webcam is listed with no yellow warning icon.

- Open the Windows Camera app (Start → search "Camera").

- Confirm the live video feed from the webcam appears in the preview window.

- Click the capture button to take a photo.

- Open the Photos/Camera Roll folder to confirm the captured image was saved successfully.

Verification: Live feed visible in the Camera app + a saved photo file confirms the webcam is fully working.

Task 2 — Set up a local printer and print a test page

- Connect the printer via USB cable (or ensure it's joined to the same Wi-Fi network for a network printer).

- Go to Settings → Devices → Printers & scanners → Add a printer or scanner.

- Windows searches for available printers; select the detected printer from the list.

- Let Windows install the driver automatically, or insert the manufacturer's driver CD/download it from their website if it isn't auto-detected.

- Once installed, click on the printer → Manage → Print a test page.

- Confirm the test page prints correctly, with proper alignment and no streaks or missing colours.

Verification: A physical test page comes out of the printer, and the printer shows status "Ready" in Settings.

Task 3 — Simulate and resolve "printer not detected"

- Unplug the printer's USB cable while the computer is on.

- Open Device Manager → Printers (or Universal Serial Bus controllers) and observe: the printer entry either disappears or shows a yellow warning triangle, indicating Windows can no longer communicate with it.

- In Printers & scanners settings, the printer's status changes to "Offline" or "Not available."

- Reconnect the USB cable.

- In Device Manager, click Action → Scan for hardware changes. This forces Windows to re-detect connected devices without needing a restart.

- Confirm the printer reappears in Device Manager with no warning icon, and its status in Printers & scanners returns to "Ready."

- If it doesn't reconnect automatically, right-click the printer in Device Manager and select "Update driver," or remove and re-add the printer from Printers & scanners.

Verification: Printer listed with no error icon in Device Manager, and status shows "Ready" instead of "Offline."

Task 4 — Webcam works in Camera app but not in Zoom

- Since it already works in the Camera app, the webcam itself is fine — the problem is with Zoom or a setting.

- Close any other application that might be holding onto the webcam (Skype, Teams, browser tabs with camera access, OBS, etc.), since only one app can use a webcam at a time and a background app can silently block Zoom from accessing it.

- Open Settings → Privacy → Camera and make sure "Allow apps to access your camera" is turned on, and specifically that Zoom is toggled on in the list of allowed apps.

- Open Zoom → Settings → Video and check the camera dropdown — if it's set to "None" or the wrong device, select the correct webcam from the list.

- Update Zoom to the latest version, since outdated versions sometimes lose camera permissions after a Windows update.

- Fully close Zoom (check it's not still running in the background) and open it again to test.

Verification: The webcam now shows video inside a Zoom meeting.