Task 1: VLAN Network Diagram

VLAN 10 (Streaming / IoT Media):

- Device: Smart TV connected to Switchport FastEthernet0/1

- Subnet: 192.168.10.0/24

VLAN 20 (Personal Devices):

- Device 1: User Laptop connected to Switchport FastEthernet0/2

- Device 2: Friend's Phone (via Access Point bridging to Switchport FastEthernet0/3)

- Subnet: 192.168.20.0/24

Task 2: Configuring two VLANs on a simulated switch

1. Topology & IP Scheme

- VLAN 10 (Faculty / Lab 1): Subnet 192.168.10.0/24

  - PC1: 192.168.10.11 connected to Fa0/1

  - PC2: 192.168.10.12 connected to Fa0/2

  - PC3: 192.168.10.13 connected to Fa0/3

- VLAN 20 (Students / Lab 2): Subnet 192.168.20.0/24

  - PC4: 192.168.20.14 connected to Fa0/4

  - PC5: 192.168.20.15 connected to Fa0/5

2. Cisco Switch CLI Configuration Commands

Switch> enable
Switch# configure terminal

! --- Create VLANs ---
Switch(config)# vlan 10
Switch(config-vlan)# name Faculty_VLAN
Switch(config-vlan)# exit

Switch(config)# vlan 20
Switch(config-vlan)# name Student_VLAN
Switch(config-vlan)# exit

! --- Assign Ports to VLAN 10 ---
Switch(config)# interface range FastEthernet0/1 – 3
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 10
Switch(config-if-range)# exit

! --- Assign Ports to VLAN 20 ---
Switch(config)# interface range FastEthernet0/4 – 5
Switch(config-if-range)# switchport mode access
Switch(config-if-range)# switchport access vlan 20
Switch(config-if-range)# exit
Switch(config)# end

3. Verification & Connectivity Testing

- Verification: show vlan brief

- Intra-VLAN Ping (Within VLAN 10):

  - From PC1 terminal: ping 192.168.10.12

  - Result: Successful replies (0% packet loss).

- Inter-VLAN Ping (Across VLAN 10 to VLAN 20):

  - From PC1 terminal: ping 192.168.20.14

  - Result: Request timed out (100% packet loss).

Reason: VLANs create separate logical networks. Communication between different VLANs requires a router or Layer 3 switch.

Task 3: Moving a PC from VLAN 10 to VLAN 20

Scenario: PC3 (connected to interface FastEthernet0/3) is moved from VLAN 10 to VLAN 20.

Step 1: Before State Configuration

Check port assignment: Switch# show vlan brief

Output before change:
Plaintext
VLAN Name                             Status    Ports
---- -------------------------------- --------- -------------------------------
10   Faculty_VLAN                     active    Fa0/1, Fa0/2, Fa0/3
20   Student_VLAN                     active    Fa0/4, Fa0/5

- Connectivity Test: PC3 (192.168.10.13) can ping PC1 (192.168.10.11), but cannot ping PC4 (192.168.20.14).

Step 2: Configuration Reassignment

Switch# configure terminal
Switch(config)# interface FastEthernet0/3
Switch(config-if)# switchport access vlan 20
Switch(config-if)# exit
Switch(config)# end
Switch# do wr

Step 3: Device IP Reconfiguration

On PC3, change the static IP address to match the new subnet:

- Old IP: 192.168.10.13 / 255.255.255.0

- New IP: 192.168.20.13 / 255.255.255.0

Step 4: After State Verification

Switch# show vlan brief

Output after change:
Plaintext
VLAN Name                             Status    Ports
---- -------------------------------- --------- -------------------------------
10   Faculty_VLAN                     active    Fa0/1, Fa0/2
20   Student_VLAN                     active    Fa0/3, Fa0/4, Fa0/5

- Connectivity Test:

  - Ping from PC3 (192.168.20.13) to PC4 (192.168.20.14): Successful (Reply received)

  - Ping from PC3 (192.168.20.13) to PC1 (192.168.10.11): Fails (Request timed out)

Task 4: Assigning 10 ports to VLAN 30 using Automated Port Configuration

Switch# configure terminal
Switch(config)# vlan 30 
Switch(config-vlan)# name Guest_Network 
Switch(config-vlan)# exit

! Batch configure ports FastEthernet0/1 through FastEthernet0/10 
Switch(config)# interface range FastEthernet0/1 - 10 
Switch(config-if-range)# switchport mode access 
Switch(config-if-range)# switchport access vlan 30 
Switch(config-if-range)# spanning-tree portfast 
Switch(config-if-range)# no shutdown 
Switch(config-if-range)# exit 
Switch(config)# end 
Switch# do wr