Task 1: Building the Network Topology (Home WIFI Network.pkt)

Devices Used:

- 1× Wireless Router: WRT300N (Generic Wireless Home Gateway)

- 3× Workstations: PC-PT (PC0, PC1, PC2)

  - PC0 & PC1: Connected via standard Cat6 copper straight-through cables.

  - PC2 (Wireless Client): Equipped with a WMP300N wireless NIC module to connect over Wi-Fi.

Step-by-Step Construction Steps:

- Open Cisco Packet Tracer and save the blank workspace as HomeWiFiNetwork.pkt.

- Under Network Devices → Wireless Devices, drag a WRT300N wireless router onto the canvas.

- Under End Devices, drag 3 generic PCs onto the canvas (PC0, PC1, PC2).

- Wired Connections (PC0 & PC1):

- Select Connections → Copper Straight-Through.

- Connect PC0 (FastEthernet0) to Router Ethernet port 1 (Ethernet 1).

- Connect PC1 (FastEthernet0) to Router Ethernet port 2 (Ethernet 2).

- Wireless Module Setup (PC2):

- Click PC2 → Physical tab.

- Toggle the power switch OFF.

- Drag the default copper PT-HOST-NM-1CFE module off into the bottom module tray.

- Drag the WMP300N wireless adapter module into the empty slot.

- Toggle the power switch back ON.

- Open the Desktop tab → PC Wireless → Connect tab → select the router's SSID (Home_Network or default Default) and click Connect.

Task 2: Configure Static IP Addresses

Subnet Plan:

- Network Subnet: 192.168.10.0/24

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.10.1

Device Configuration Breakdown:

| Device | Interface | IP Address | Subnet Mask | Default Gateway | DNS Server |
| --- | --- | --- | --- | --- | --- |
| Router (WRT300N) | LAN / Internal | 192.168.10.1 | 255.255.255.0 | N/A | 1.1.1.1 |
| PC0 | FastEthernet0 | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 | 1.1.1.1 |
| PC1 | FastEthernet0 | 192.168.10.20 | 255.255.255.0 | 192.168.10.1 | 1.1.1.1 |
| PC2 | Wireless0 (WMP300N) | 192.168.10.30 | 255.255.255.0 | 192.168.10.1 | 1.1.1.1 |

Configuration Steps on Devices:

- Router LAN IP:

- Open WRT300N → GUI tab → Setup → Basic Setup.

- Set IP Address to 192.168.10.1 and Subnet Mask to 255.255.255.0.

- Under DHCP Server Setting, change to Disabled (to enforce static IP testing) and click Save Settings at the bottom.

- PC IP Settings (PC0, PC1, PC2):

- Click on the PC → Desktop tab → IP Configuration.

- Toggle Static.

- Enter the corresponding IP, Subnet Mask (255.255.255.0), and Default Gateway (192.168.10.1).

Task 3: Connectivity Verification (Ping Testing)

After configuring the IP addresses, use the Command Prompt on each PC.

- From PC0 (192.168.10.10) to Default Gateway (192.168.10.1):

C:\> ping 192.168.10.1

Pinging 192.168.10.1 with 32 bytes of data:
Reply from 192.168.10.1: bytes=32 time=1ms TTL=255
Reply from 192.168.10.1: bytes=32 time=1ms TTL=255
Reply from 192.168.10.1: bytes=32 time=1ms TTL=255
Reply from 192.168.10.1: bytes=32 time=1ms TTL=255

Ping statistics for 192.168.10.1:
    Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

- From PC0 (192.168.10.10) to PC1 (192.168.10.20 - Wired):

C:\> ping 192.168.10.20

Pinging 192.168.10.20 with 32 bytes of data:
Reply from 192.168.10.20: bytes=32 time<1ms TTL=128
Reply from 192.168.10.20: bytes=32 time<1ms TTL=128
Reply from 192.168.10.20: bytes=32 time<1ms TTL=128
Reply from 192.168.10.20: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.10.20:
    Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

- From PC0 (192.168.10.10) to PC2 (192.168.10.30 - Wireless):

C:\> ping 192.168.10.30

Pinging 192.168.10.30 with 32 bytes of data:
Reply from 192.168.10.30: bytes=32 time=2ms TTL=128
Reply from 192.168.10.30: bytes=32 time=2ms TTL=128
Reply from 192.168.10.30: bytes=32 time=1ms TTL=128
Reply from 192.168.10.30: bytes=32 time=1ms TTL=128

Ping statistics for 192.168.10.30:
    Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

Task 4: Documentation of the Practical

Step 1 – Create the Project

Cisco Packet Tracer was opened and a new project was created. One wireless router and three PCs were added to the workspace.

Step 2 – Connect the Devices

Each PC was connected to an Ethernet LAN port on the wireless router using a Copper Straight-Through cable.

Step 3 – Configure the Router

The router's LAN IP address was configured as: 192.168.10.1
This address was used as the default gateway for all three PCs.

Step 4 – Configure PC1

PC1 was assigned: 192.168.10.10/24
Default gateway: 192.168.10.1

Step 5 – Configure PC2

PC2 was assigned: 192.168.10.11/24
Default gateway: 192.168.10.1

Step 6 – Configure PC3

PC3 was assigned: 192.168.10.12/24

Default gateway: 192.168.10.1

Step 7 – Test Connectivity

The Command Prompt on each PC was used to ping the other PCs and the router. The ping tests were successful, confirming that all devices were correctly configured and could communicate on the same network.

Final Network Configuration

Network:          192.168.10.0/24
Subnet Mask:      255.255.255.0
Default Gateway:  192.168.10.1

PC1:              192.168.10.10
PC2:              192.168.10.11
PC3:              192.168.10.12

Conclusion

A small home network was successfully created in Cisco Packet Tracer using one wireless router and three PCs. Static IP addresses were assigned to all PCs in the same subnet. Connectivity was tested using the ping command, and all devices were able to communicate successfully.