Task 1: Assign a Static IPv6 Address via Command Line

Windows

- Open Command Prompt as Administrator and find the network adapter: ipconfig

- Identify your active network interface name: netsh interface ipv6 show interfaces

- Assign the static IPv6 address: netsh interface ipv6 add address "Wi-Fi" 2001:db8:acad:1::10/64

- Verify the configured address: ipconfig

Linux (Ubuntu/Debian)

- Find your network interface name (e.g., eth0 or wlan0): ip -6 addr show

- Assign the static address: sudo ip -6 addr add 2001:db8:acad:1::10/64 dev eth0

- Verify the configuration: ip -6 addr show dev eth0

Task 2: Configure a Unique Local Address (ULA) IPv6 & Verify Connectivity

Windows

- Assign it: netsh interface ipv6 add address "Wi-Fi" fd00:1234:5678:1::10/64

- Verify: ipconfig

- If another LAN device has: fd00:1234:5678:1::20

- Test connectivity: ping -6 fd00:1234:5678:1::20

If replies are received, the IPv6 connection is working.

Linux

- Verify: ip link show

- sudo ip -6 addr add 2001:db8::10/64 dev eth0

- Verify: ip -6 addr show dev eth0

Task 3: Temporary IPv6 Privacy Addresses & Reboot Behaviour

- OBJECTIVE:

- Enable RFC 4941 Privacy Extensions, verify temporary SLAAC address creation, and test whether the temporary address rotates upon network disconnect/reboot.

- COMMANDS EXECUTED (Windows):

- Check current state: netsh interface ipv6 show privacy

- INITIAL ADDRESS STATE:

- IPv6 Address 			: 2001:db8:1:a:2a1:b2c3:d4e5:f678

- Temporary IPv6 Address		: 2001:db8:1:a:94c8:71be:34df:1290 (Preferred)

- DISCONNECT / RECONNECT / REBOOT OBSERVATION:

- Action: Disabled and re-enabled the adapter (and rebooted system).

- New State: Temporary IPv6 Address	: 2001:db8:1:a:b812:d340:8751:41ae (Preferred)

Temporary IPv6 Address	: 2001:db8:1:a:94c8:71be:34df:1290 (Deprecated)

- CONCLUSION:

- The system creates randomized temporary interface identifiers for outgoing internet connections to prevent persistent device tracking. Upon system reboot or expiration of the preferred lifetime, a new pseudo-random 64-bit host identifier is generated, moving the prior temporary address to a 'Deprecated' state.

Task 4: Zomato-Style IPv6 Restaurant Search Script

restaurants = [

{"name": "Haldiram's Express", "ipv6": "2001:db8:zomato:1::101"},

{"name": "Bikanervala Sweets", "ipv6": "2001:db8:zomato:1::102"},

{"name": "Barbeque Nation", "ipv6": "2001:db8:zomato:2::201"},

{"name": "Domino's Pizza Sector 18", "ipv6": "2001:db8:zomato:2::202"},

{"name": "Biryani By Kilo", "ipv6": "2001:db8:zomato:3::301"},

{"name": "Mainland China", "ipv6": "2001:db8:zomato:3::302"}

]

def search_restaurant():

query = input("Enter restaurant name (or partial name): ").strip().lower()

matches = [r for r in restaurants if query in r["name"].lower()]

if matches:

print(f"\nFound {len(matches)} matching result(s):")

print("-" * 55)

for item in matches:

print(f"Name : {item['name']}")

print(f"IPv6 : {item['ipv6']}")

print("-" * 55)

else:

print("\nNo restaurant matches found for your query.")

if __name__ == "__main__":

search_restaurant()

Sample Run:

Enter restaurant name (or partial name): Biryani

Found 1 matching result(s):

-------------------------------------------------------

Name : Biryani By Kilo

IPv6 : 2001:db8:zomato:3::301

-------------------------------------------------------

Task 5: AI-generated IPv6 subnetting scenario

Here's a scenario (3 subnets, base prefix 2001:db8:abcd::/48):

Subnet 1 - Sales:       2001:db8:abcd:1::/64
Subnet 2 - Engineering: 2001:db8:abcd:2::/64
Subnet 3 - Support:     2001:db8:abcd:3::/64

Manually assigned address from the Engineering subnet: 2001:db8:abcd:2::10/64

Configuration: : netsh interface ipv6 add address "Wi-Fi" 2001:db8:abcd:2::10/64 dev eth0

Template of text file:

IPv6 Subnetting Scenario

=========================

Verification output: Ipconfig

C:\> ipconfig /all

Ethernet adapter Ethernet:

Connection-specific DNS Suffix  . :

Description . . . . . . . . . . . : Intel(R) Ethernet Connection I219-V

Physical Address. . . . . . . . . : 00-1A-2B-3C-4D-5E

DHCP Enabled. . . . . . . . . . . : No

Autoconfiguration Enabled . . . . : Yes

IPv6 Address. . . . . . . . . . . : 2001:db8:abcd:2::10(Preferred)

Link-local IPv6 Address . . . . . : fe80::a00:27ff:fe4e:66a1%12(Preferred)

Default Gateway . . . . . . . . . : fe80::1%12

DNS Servers . . . . . . . . . . . : 2001:4860:4860::8888

NetBIOS over Tcpip. . . . . . . . : Disabled