Task 1: Common Desktop PC Hardware Issues

| No. | Hardware Issue | Typical Symptom |
| --- | --- | --- |
| 1 | Faulty RAM | PC may fail to boot, show a blank screen, or produce POST beep/error codes. |
| 2 | Failing Power Supply | PC may not start, randomly shut down, or restart unexpectedly. |
| 3 | Faulty GPU | No display, graphical artifacts, crashes, or display driver failures. |
| 4 | Failing HDD/SSD | Slow boot, missing drive, operating-system errors, or inability to boot. |
| 5 | Overheating CPU | System becomes very hot, slows down, shuts down, or restarts under heavy load. |

Task 2: Laptop Not Charging

Possible causes

- Faulty charger

- Damaged charging cable

- Damaged charging port

- Failed battery

- Internal motherboard/power-circuit fault

Troubleshooting Plan Using Basic Tools

- Check the power outlet

  - Plug another device into the same outlet. This confirms whether the electrical outlet is working.

- Inspect the charger

  - Check the cable, plug, adapter and connector for physical damage such as cuts, exposed wires, burns or loose connections.

- Check the charging port

  - Inspect the laptop's charging port for dust, bent contacts or physical damage.

  - Make sure the charger connector fits securely.

- Check the charger output

  - Use a multimeter set to DC voltage.

  - Measure the charger's output according to the adapter's rated voltage and connector polarity.

  - Compare the measured value with the rating printed on the charger.

- Perform a basic power reset

  - Disconnect the charger.

  - If the battery is removable, remove it and try powering the laptop directly from the charger - if it powers on, the battery itself may be faulty.

  - Hold the power button for approximately 15–20 seconds.

  - Reconnect the charger and try powering on the laptop.

- Test with a known-good compatible charger

  - If available, use a charger with the correct voltage, connector and sufficient current rating.

  - If the laptop charges with the replacement charger, the original charger is likely faulty.

Task 3: Test a USB Phone Charger

Procedure

- Set the multimeter to DC Voltage (20V range).

- Plug the USB phone charger into a power outlet.

- Identify the +5V and Ground (GND) pins of the USB connector.

- Place the black probe of the multimeter on the Ground (GND) pin.

- Place the red probe on the +5V pin.

- Read the voltage displayed on the multimeter.

- Record the measured voltage.

- Compare the measured value with the standard USB voltage range

Result

Expected USB voltage:  4.75V – 5.25V

Measured voltage:  5.02V

Result:  PASS

Task 4: Test the PSU 12V Rail

Procedure

- Shut down the PC.

- Switch off the PSU.

- Disconnect the AC power cable.

- Disconnect the PSU's ATX connector from the motherboard.

- Set the multimeter to DC voltage.

- Locate the yellow wire (+12 V) and black wire (GND) on the appropriate disconnected ATX power connector.

- Place the red probe on the yellow wire's terminal and the black probe on a black-wire terminal.

- Record the voltage.

Result

Expected nominal voltage: 12V

Measured voltage:  12.08V

Result:  Within expected range

If the voltage is below 12 V

This usually means the power supply is weak or failing and may not be able to properly power components like the GPU — it should be replaced or tested further.