Task 1: Reseating RAM

- Turn off the PC and unplug it.

- Open the case and locate the RAM slots on the motherboard.

- Push the retention clips on both ends of the RAM stick outward to release it.

- Pull the RAM stick straight out.

- Inspect the module and slot for visible dust or damage.

- Align the notch on the RAM module with the slot.

- Press the module firmly into the slot until the retaining clips lock.

- Close the case and boot up to confirm it still works.
(Note any problems you noticed, like a clip that felt stiff or dust buildup around the slot.

- Close the case and reconnect the power cable.

- Start the computer and Check whether the PC successfully boots.

Result:

Before reseating: PC failed to display/POST.

Action: RAM removed and reinserted.

After reseating: PC successfully completed POST and booted.

Task 2: Faulty GPU Troubleshooting Checklist

GPU Troubleshooting Checklist

- Identify the symptom
Check whether the problem is no display/signal, artifacts, crashes, black screen, or poor performance.

- Check the display cable
Ensure HDMI/DisplayPort is connected securely to the correct GPU output.

- Check GPU power
Verify that all required PCIe 6/8-pin or 6+2-pin power connectors are properly connected.

- Reseat the GPU
Power off the PC and reinstall the graphics card securely in the PCIe slot.

- Check Device Manager
Look under Display adapters for the graphics card and check for warning icons or "Code 43" error.

- Check/update the graphics driver
Install the appropriate driver from the GPU manufacturer.

- Test the GPU
Run a suitable graphics workload or game and monitor for crashes, artifacts, or abnormal behavior.

- Confirm the fix
Verify stable display output and normal operation under load. Confirm that the display works correctly after the fix.

Task 3: Simulate a Storage Problem

PROCEDURE: -

- Turn off the computer.

- Disconnect the power cable.

- Open the computer case.

- Disconnect the SATA cable from the HDD/SSD (or remove the NVMe SSD).

- Turn on the computer.

BEFORE RECONNECTING: -

- No Boot Device Found

- Operating System Not Found

- Boot Device Not Detected

- Storage drive not listed in BIOS

- Turn off the computer again.

- Reconnect the SATA cable or reinstall the NVMe SSD.

- Restart the computer.

AFTER RECONNECTING: -

- The BIOS detects the SSD/HDD.

- The operating system starts normally.

Task 4: PC Randomly Restarts - Which Component to Check First?

First, I check RAM because RAM is one of the most common causes of random restarts. A loose or faulty RAM module can lead to system crashes, restart loops, or blue screen errors.

STEPS TO RESOLVE PROBLEM: -

- Turn off the computer and disconnect the power.

- Open the computer case.

- Remove the RAM module.

- Check for dust or damage.

- Reinsert the RAM securely.

- If there are multiple RAM modules, test one module at a time.

- Test different recommended RAM slots if appropriate.

- Restart the computer and observe whether the restart issue is resolved.

- If the problem remains, continue testing the GPU, storage, PSU, CPU temperature, and other components.