Task 1 — Laptop won't power on even when plugged in

Possible causes

- Faulty charger

- Damaged charging port

- Failed battery

- Power button problem

- Internal motherboard/power-circuit fault

- Check the charging light on the laptop — if it's off, check the charger and outlet first.

- Try a different known-working charger, to rule out a dead charger.

- Hold the power button for 15-20 seconds (a "hard reset") to drain any static charge, then try again.

- Remove the battery (if removable) and try running on charger power alone.

- Listen/watch for any signs of life (fan spinning, lights, sounds) even if the screen stays black — this tells you if it's a display problem instead of a power problem.

Task 2 — Monitor blank, but CPU seems to be running

- Check the monitor is turned on and set to the correct input source (HDMI/VGA/DisplayPort).

- Check the cable is firmly connected at both ends.

- Try a different cable or a different monitor, if available.

- If using a graphics card, make sure the monitor cable is plugged into the GPU, not the motherboard.

- Restart the computer.

- Press Windows + P and select Duplicate or PC Screen Only.

- Listen for beep codes, which can point to a RAM or GPU issue even without display.

- If nothing works, reseat the RAM and GPU, since a loose connection can cause exactly this symptom.

Task 3: Wireless Mouse Not Working

- Try the mouse on a different computer — if it still doesn't work, the mouse itself is faulty.

- Try a different wireless mouse on the same USB port/receiver — if that works, the original mouse is the problem.

- Move the USB receiver to a different port, in case that specific port is faulty.

- Check Device Manager for a warning icon on the mouse or USB receiver.

- Check the mouse's batteries and power switch (many wireless mice have an on/off switch on the bottom).

Task 4 — Gaming cafe PC overheating and shutting down

- Dust inside the Computer: Open the cabinet and clean the dust using compressed air or a soft brush.

- CPU or GPU fan not working: Check whether the fan is spinning properly. Replace it if it is not working.

- Dry or old thermal past: Remove the old thermal paste and apply new thermal paste between the CPU and heatsink.