Task 1: IPv4 Address Classes & Real-World Use Cases

| Class | IP Address Range | Default Subnet Mask | Purpose / Primary Scope | Real-World Example |
| --- | --- | --- | --- | --- |
| A | 1.0.0.0 – 126.255.255.255 | 255.0.0.0 (/8) | Massive global networks with millions of hosts | Cloud backbones and hyper-scale organizations (e.g., Apple's public 17.0.0.0/8 block or AWS internal networks using 10.0.0.0/8). |
| B | 128.0.0.0 – 191.255.255.255 | 255.255.0.0 (/16) | Medium to large enterprise and educational networks | Large university campuses (e.g., allocating a 172.16.0.0/16 block for student Wi-Fi and department labs). |
| C | 192.0.0.0 – 223.255.255.255 | 255.255.255.0 (/24) | Small business and local home networks | SOHO (Small Office/Home Office) Wi-Fi routers assigning addresses in the 192.168.1.0/24 range to phones and laptops. |
| D | 224.0.0.0 – 239.255.255.255 | N/A (No host bits) | Multicast streaming and routing discovery | Live video streaming (e.g., IPTV feeds) or routing protocol broadcasts (e.g., OSPF hello packets to 224.0.0.5). |
| E | 240.0.0.0 – 255.255.255.255 | N/A (Experimental) | Research, development, and future reserve | IETF laboratory experiments, future IP protocol development, and test environments. |

Task 2: Network calculation for 192.168.10.45 / 255.255.255.0

GIVEN:

- IP Address = 192.168.10.45

- Subnet Mask = 255.255.255.0 (/24)

Step 1: Network Address

- The last part becomes 0.

- Network Address = 192.168.10.0

Step 2: Broadcast Address

- The last part becomes 255.

- Broadcast Address = 192.168.10.255

Step 3: Valid Host Range

- Everything between network and broadcast address

- First usable IP: 192.168.10.1

- Last usable IP: 192.168.10.254

| Item | Answer |
| --- | --- |
| Network Address | 192.168.10.0 |
| First Host | 192.168.10.1 |
| Last Host | 192.168.10.254 |
| Broadcast Address | 192.168.10.255 |

- Base Network: 10.0.0.0/24 (Subnet mask: 255.255.255.0)

- Number of subnets required: 4

- Borrowed bits formula: 2n≥4⟹n=2 bits.

- New Prefix Length: 24+2=/26

- New Subnet Mask:

  - Last octet binary: 110000002=128+64=192

  - Mask: 255.255.255.192

- Block Size (Increment): 256-192=64 addresses per subnet.

- Usable hosts per subnet: 26-2=64-2=62 hosts.

| Group Name | Network Address | Subnet Mask | Usable Host Range | Broadcast Address |
| --- | --- | --- | --- | --- |
| Delivery Partners | 10.0.0.0/26 | 255.255.255.192 | 10.0.0.1 – 10.0.0.62 | 10.0.0.63 |
| Restaurants | 10.0.0.64/26 | 255.255.255.192 | 10.0.0.65 – 10.0.0.126 | 10.0.0.127 |
| Customers | 10.0.0.128/26 | 255.255.255.192 | 10.0.0.129 – 10.0.0.190 | 10.0.0.191 |
| Admins | 10.0.0.192/26 | 255.255.255.192 | 10.0.0.193 – 10.0.0.254 | 10.0.0.255 |

Task 4: College Wi-Fi Subnetting Scenario (172.16.0.0/16)

Constraints & Requirements:

- Starting Network: 172.16.0.0/16

- Required Subnets: At least 20 subnets.

Step-by-Step Calculation:

- Find Borrowed Subnet Bits (n):

- 2n≥20

- For n=4⟹24=16 (insufficient)

- For n=5⟹25=32≥20

- Borrow 5 bits from the host portion.

- Calculate New Subnet Mask:

  - Starting prefix: /16

  - New prefix: 16+5=/21

  - Binary representation: 11111111.11111111.11111000.00000000

  - Third octet value: 128+64+32+16+8=248

  - New Subnet Mask: 255.255.248.0

- Calculate Usable Hosts per Subnet:

  - Total host bits remaining: 32-21=11 bits.

  - Usable hosts formula: 2h-2

  - 211-2=2048-2=2,046 hosts per subnet.

Final Answer:

- New Subnet Mask: 255.255.248.0 (CIDR: /21)

- Subnets Created: 32 subnets (exceeds the 20 requested).

- Usable Hosts Available per Subnet: 2,046 hosts.

- The Problem:

An IPv4 address consists of four numbers (octets) separated by periods. Each octet is made up of exactly 8 bits, meaning the decimal value of any single octet can only range from 0 to 255 (28-1). The value 300 exceeds the 8-bit boundary and cannot be represented in binary, making it an invalid IP address that network drivers and operating systems will reject.

- What a Valid IP Address Looks Like:

In a standard home/office network subnet (192.168.1.0/24 with subnet mask 255.255.255.0):

  - 192.168.1.0 is reserved for the network ID.

  - 192.168.1.255 is reserved for the broadcast address.

  - A valid, assignable host address must fall within the range of 192.168.1.1 to 192.168.1.254 (for example: 192.168.1.50).