CompTia A+  Assessment Code AP-A1

Hardware Components & System Assembly

Section A — Concept Application

S1 SCENARIO: Selecting Compatible Hardware for a new Office Workstation.

Question: Explain the process of selecting a compatible motherboard, CPU, RAM, and storage device for a new office workstation, and justify how you would confirm they will work together.

Choosing compatible motherboard, CPU, RAM, storage for a new office workstation I Pick the CPU first based on the budget and how much power office work actually needs (a mid-range CPU is usually enough). Then pick a motherboard whose CPU socket matches that CPU exactly (e.g., LGA1700 for certain Intel chips, AM5 for certain AMD chips). Check the motherboard's spec sheet for which RAM type (DDR4/DDR5) and speeds it supports, and pick RAM that matches. For storage, I check if the board has an M.2 slot for NVMe (faster) or just SATA ports.

To confirm they'll work together: check the motherboard maker's website for the CPU support list, check the RAM against their "Qualified Vendor List" (QVL), and make sure the motherboard's physical size (ATX/mATX) fits the case.

Compatibility checklist

| Priority | Component | Compatibility Check |
| --- | --- | --- |
| 1 | CPU | Socket and motherboard/BIOS support |
| 2 | Motherboard | CPU socket, chipset, RAM type, storage interfaces |
| 3 | RAM | DDR type, capacity, speed and motherboard support |
| 4 | Storage | SATA or M.2 interface and motherboard support |
| 5 | Form Factor Compatibility | Verify the motherboard standard (ATX, Micro-ATX, or Mini-ITX) matches the chassis mounting standoffs and PSU clearance. |

S2 SCENARIO: HDD, SSD, NVMe Analysis & Recommendation which is best for office work.

Question: Explain the differences between HDD, SSD, and NVMe storage, and recommend the most suitable option for this workload, giving technical reasons for your choice.

For office/productivity work (documents, browsers, email), a SATA SSD is the best pick — it gives a huge speed boost over HDD for boot and app loading, and office tasks don't actually need NVMe's extra speed, so it's not worth the extra cost here.

| Storage Type | HDD (Hard Disk Drive) | SATA SSD | NVMe SSD |
| --- | --- | --- | --- |
| Physical Mechanism | Mechanical spinning magnetic platters with moving read/write heads | Solid-state NAND flash memory (no moving parts) | High-speed NAND flash connected directly to PCIe bus |
| Typical Read/Write Speed | 80 – 160 MB/s | 500 – 550 MB/s | 3,500 – 7,500+ MB/s |
| Latency | High (~10–15 ms) | Low (<0.1 ms) | Ultra-Low (<0.02 ms) |
| Form Factor & Interface | 3.5" or 2.5" SATA III | 2.5" SATA III or M.2 SATA | M.2 (2280) via PCIe 4.0/5.0 |
| Typical use | Bulk storage | Office systems | High-performance systems |

S3 SCENARIO: GPU and cooling for sustained Graphic & design work.

Question: Explain the role of the GPU and the cooling system in maintaining performance and stability under this kind of sustained load.

A GPU (Graphics Processing Unit) processes graphical workloads and helps the workstation run graphics-intensive applications efficiently. For a workstation running mildly graphics-intensive design software for several hours, an appropriate GPU handles rendering the graphics/design work instead of the CPU, which keeps the software running smoothly — it needs enough VRAM (Video Random Access Memory) for the software being used.

The cooling system removes heat generated by the CPU, GPU, and other components during operation. Under sustained workloads, effective cooling prevents excessive temperatures and helps maintain system stability and performance.

A typical cooling system can include: CPU heatsink, CPU fan, GPU cooler, Case fans, Thermal paste, Airflow through the computer case

Without adequate cooling, components may reach high temperatures, causing reduced performance, instability, crashes, or shutdowns.

S4 SCENARIO: Hardware Compatibility Risks in Multi-Vendor Builds.

Question: Explain why hardware compatibility matters when assembling a computer, and identify potential issues that could arise from incompatible components.

Hardware compatibility is important because computer components must communicate correctly and physically fit together.

For example, a CPU must match the motherboard's supported CPU socket and chipset. RAM must match the motherboard's supported memory technology, while storage must use an interface supported by the motherboard.

Problems caused by incompatible components

- CPU cannot be installed in the motherboard socket.

- RAM may not fit or may not be supported.

- Storage device may not have a compatible interface.

- GPU may not physically fit in the case.

- PSU may not provide the required connectors or power.

- System may fail to boot or complete POST.

- Hardware may operate incorrectly or cause instability.

Therefore, compatibility should be verified using the manufacturer's specifications before assembling the system.

S5 SCENARIO: Choosing PSU wattage + safety checks before powering on

Question: Explain how you would choose an appropriately rated PSU for this build and describe the safety and connection checks you would carry out before powering it on.

When selecting an appropriate rated PSU I first of all Add up the estimated wattage of the CPU, GPU, drives, and fans, then add about 20–30% extra headroom, and pick a PSU rated for that total (an 80+ certified one is more efficient). Make sure it has the connectors you need: 24-pin, CPU 4/8-pin, GPU 6/8-pin, and SATA power.
Before powering on: I make sure the PC is unplugged while connecting cables, double-check every connector is fully seated, check there are no loose screws touching the motherboard (short-circuit risk), manage cables so nothing is pinched or blocking a fan, then plug in and power on.

S6 SCENARIO: Confirming a system passes POST, and what to do if it doesn't

Question: Explain the checks you would perform to confirm the system passes POST successfully, and what you would do if it did not.

- Checks: fans spin and lights turn on, the monitor shows the manufacturer/BIOS splash screen, no beep codes or on-screen errors, and in BIOS the CPU, correct RAM amount, and storage drive are all detected.

- If it fails: listen for beep codes (they usually point to a specific part), reseat the RAM and GPU, double-check all power cables are connected, confirm the CPU cooler is mounted properly, and if needed strip the build down to just CPU + one RAM stick to isolate which part is causing the failure.

Section B — Practical Coding Tasks

Task 1: Component Identification & Functions

The core components of the desktop system were identified and their functions documented.

| Component | Make & Model (Sample Specification) | Primary Function PDF | Key Physical / Electrical Attribute |
| --- | --- | --- | --- |
| Motherboard | ASUS Prime B760M-A Wi-Fi | Connects and allows communication across all hardware. | Intel LGA 1700, 4x DDR5 DIMM, PCIe 4.0 x16 |
| CPU | Intel Core i5-14400 (10 Cores, 4.7 GHz) | Executes system instructions, logical operations, and OS tasks. | 65 W Base TDP, Integrated Intel UHD Graphics 730 |
| RAM | Kingston Fury Beast 16 GB (2x8GB) | High-speed temporary workspace holding active program data. | DDR5-5200 MT/s, CL40, 1.1 V |
| GPU | NVIDIA GeForce RTX 4060 8 GB | Processes 3D geometry, shaders, video decoding, and display pipelines. | PCIe 4.0 x8 interface, 115 W TGP, GDDR6 |
| Storage | Crucial P3 Plus 1 TB NVMe SSD | Non-volatile primary storage for OS, applications, and datasets. | M.2 2280, PCIe Gen4 x4, up to 5000 MB/s read |
| Cooling | DeepCool AK400 Air Cooler | Dissipates thermal energy from the CPU die into the chassis airflow. | 4 copper heat pipes, 120mm PWM fan, 220 W TDP rating |
| PSU | Corsair CX650M 650 W | Converts 110–240 V AC to regulated +12V, +5V, and +3.3V DC power. | 80-PLUS Bronze, Semi-Modular |

Task 2: Motherboard Ports, Slots & Socket Identification

- CPU Socket Details: Intel LGA 1700 (Land Grid Array featuring 1,700 contact pins on the socket). Compatible with Intel 12th, 13th, and 14th Generation Core i3/i5/i7/i9 desktop processors.

- Expansion Slots:

  - 1x PCIe 4.0 x16 slot directly wired to CPU for dedicated graphics cards.

  - 1x PCIe 3.0 x1 slot for add-in network or sound cards.

  - 2x M.2 2280 M-Key slots for high-speed PCIe NVMe solid-state storage.

- Headers & Connectors: 24-Pin ATX Main Power, 8-Pin EPS CPU Power, 4x SATA III 6 Gb/s ports, USB 3.2 Gen 1 front header, USB 2.0 headers, HD Audio header, and Front Panel header (Power SW, Reset SW, HDD LED, Power LED).

Task 3: RAM & Storage Installation Procedure

- Power off and unplug the PC, then press the power button once to drain any leftover charge.

- Electrostatic Discharge (ESD) Safety: Put on an anti-static wrist strap clipped to an unpainted metal section of the chassis frame, or touch grounded bare metal before handling components.

- RAM Removal: Push the RAM slot clips outward and pull the stick straight up to remove it.

- RAM Reinstallation: Reinsert it by lining up the notch and pressing down evenly on both ends until the clips click.

- Storage Drive Removal: disconnect the SATA data + power cables (or unscrew the tiny screw for an M.2 drive), then remove it.

- Drive Reinstallation: Reinstall by reversing those steps, making sure connections are firm.

- Power on the PC and press DEL or F2 to enter the UEFI BIOS Setup.

- Verify that the BIOS dashboard detects:

  - DRAM Status: Full 16,384 MB (16 GB) operating in Dual-Channel Mode.

  - NVMe Storage Configuration: Crucial CT1000P3PSSD8 detected with PCIe Gen4 link speed.

Task 4: Full Desktop PC Assembly

- Step 1: Base Component Preparation (Outside Case): Place the motherboard on its anti-static box. Install the CPU into the socket by aligning the notch, lower the retention arm, lock the lever, insert RAM into slots A2 and B2, and install the M.2 NVMe SSD.

- Step 2: Chassis & PSU Preparation: Install the Power Supply Unit into the bottom bay of the PC case and secure with 4 hex screws. Install the rear I/O shield and verify standoff screw locations match the motherboard form factor.

- Step 3: Motherboard Mounting: Lower the motherboard into the chassis, aligning ports with the rear I/O cutout. Secure with #6-32 motherboard screws in a cross pattern.

- Step 4: CPU Cooler Installation: Apply thermal paste (pea-sized dot), mount the heatsink bracket evenly, and plug the 4-pin PWM wire into the CPU_FAN header.

- Step 5: Cable Routing & Connections: Connect the 24-pin ATX power, 8-pin CPU power, front panel power/reset switches, front USB 3.0, and HD audio cables through the chassis backplate grommets.

- Step 6: GPU Installation: Insert the GPU into the primary PCIe x16 slot until the retention latch snaps, secure the metal bracket with chassis screws, and connect the 8-pin PCIe power lead.

- Step 7: Final inspection: Check RAM & GPU is seated, CPU cooler is secured, Power & Storage cables are connected, no cables are touching fans & no loose screws are inside the case.

- Step 7: Power On Self-Test (POST): Connect monitor and power cable, switch PSU to ON, press the power button, and confirm entry into UEFI BIOS screen with normal temperatures (<40°C).

Section C — Mini Capstone Project

(Mini Project: Workstation Deployment & Boot-Failure Diagnosis)

Objective: ABC Solutions is setting up a new computer lab. After assembling several systems, some computers fail to start while others display hardware detection errors, and the IT team suspects incorrect installation or compatibility issues. Build (or inspect) a workstation, diagnose the likely causes of these failures, and produce a diagnostic and installation-verification report — combining component selection, installation procedure, and compatibility troubleshooting.

- Three Components to Check First When a PC Fails to Boot

The first three components I would check are:

  - Power Supply Unit & Power Delivery Cables: Verify the PSU is turned ON, the AC power cable is live, and the 24-pin ATX, 8-pin EPS connectors and the front-panel POWER_SW connector is wired to the correct pins and that no misaligned brass standoffs. If the system shows zero LED lights or fan activity, power delivery is the root cause.

  - RAM: Improperly seated memory sticks or modules in incorrect channels will prevent POST, triggering RAM diagnostic LEDs or continuous beep codes. It is the most common cause of a no-boot.

  - I) GPU seating (if a discrete card is used) — a loose GPU causes "no display" even though the system otherwise powers on.

II) Motherboard/CPU: The motherboard connects the major components, while the CPU performs the initial processing required for system startup. Incorrect CPU installation, incompatible hardware, or incorrect power connections can prevent successful POST.

- How motherboard–CPU compatibility affects functionality

The CPU has to physically match the motherboard's socket (e.g., LGA 1700 for Intel 12th–14th Gen). But it's not just physical fit — the motherboard's BIOS also has to support that exact CPU generation, so sometimes an older motherboard needs a BIOS update before a newer CPU will even boot. Even with matching sockets, installing a newer generation processor (e.g., 14th Gen on an older B660 board) will result in a blank screen and no-POST condition unless the BIOS firmware has been updated to support the newer CPU microcode.

- RAM & Storage Installation Verification Procedure

  - Physical Check: Confirm that RAM retention latches are locked flush and M.2 SSDs are seated parallel to the PCB with screws properly fastened.

  - BIOS / POST Diagnostic Confirmation:

-In the UEFI Main Screen, check that total memory equals the sum of installed modules and verify that XMP / EXPO profiles operate at rated memory speeds.

-Under Storage Configuration / NVMe Configuration, check that the SSD manufacturer model and total storage capacity appear.

-Under the Boot Priority List, ensure the primary SSD is registered as a UEFI Boot Target.

- Hardware-installation checklist (to prevent future failures)

  - Check CPU socket, RAM type, and PSU wattage against the motherboard's official spec sheet before starting the build.

  - Follow ESD precautions (anti-static strap or grounding touch) throughout.

  - After connecting each major component, physically check the connection is fully seated before moving to the next step, instead of checking everything only at the end.

  - Do a quick test boot outside the case, on an anti-static surface, before final assembly — makes any issue easier to spot and fix.

- Why SSD over HDD for this lab's office workstations

  - SSDs load the OS and apps much faster than HDDs, which matters for a lab where people are constantly opening files and switching programs.

  - They have no moving parts, so they're more reliable in a shared lab where PCs might get bumped or moved.

  - They also use less power and run quieter — useful with a room full of machines running all day.

  - High IOPS ratings (100,000+ IOPS on SSDs vs. ~150 IOPS on HDDs) prevent system freezes when multiple background tasks—such as antivirus scanning, classroom monitoring software, and OS updates—run alongside student applications

  - Since office/productivity work doesn't need extreme speed, a standard SATA SSD (not necessarily a pricier NVMe) is the best value for this lab's use case.

Section D — AI-Augmented Learning

Step 1: Build With AI

The exact prompt(s) I gave the AI tool.

“Create a step-by-step troubleshooting checklist for a junior technician diagnosing a desktop PC that fails to boot immediately after assembly. Put the steps in the correct priority order. Include ESD precautions, motherboard, RAM, storage, and PSU as possible failure points. Make it a clear checklist or simple decision flow that can be followed on-site. Apply the checklist to the ABC Solutions scenario where newly assembled systems fail to start or display hardware detection errors”.

AI's Original Checklist

Desktop PC Boot-Failure Checklist

- Check that the PC has power and that the PSU switch is ON.

- Check the 24-pin motherboard power connector and CPU/EPS power connector.

- Check the RAM modules and reseat them if necessary.

- Check that the CPU and CPU cooler are correctly installed.

- Check the GPU and its power connectors.

- Check whether the motherboard shows POST LEDs or error codes.

- Enter BIOS/UEFI and check whether RAM is detected.

- Check whether the storage device is detected in BIOS/UEFI.

- Check SATA/M.2 connections.

- Check component compatibility, including CPU, motherboard, RAM and storage.

- Disconnect unnecessary peripherals and test the system with essential components.

- If the problem continues, test individual components to identify the faulty part.

Step 2: Test & Debug — Corrected Checklist

After reviewing the AI-generated checklist, I identified that ESD safety should be explicitly stated before opening the computer or reseating components. This is important because a technician should protect components from electrostatic discharge before handling internal hardware.

Corrected Checklist

- Confirm the reported symptom: Determine whether the PC has no power, powers on but fails POST, displays a blank screen, or reports a hardware detection error.

- Follow ESD and safety precautions: Shut down the system, disconnect AC power before handling internal components, and use appropriate ESD precautions.

- Check external power: Verify the wall outlet, power cable, PSU switch and basic external connections.

- Check PSU connections: Verify the 24-pin motherboard connector and CPU/EPS power connector. Check GPU power connectors if required.

- Check motherboard indicators: Observe POST LEDs, diagnostic displays, or beep codes if available.

- Check and reseat RAM: Verify that the RAM modules are installed correctly. Test modules individually if necessary.

- Check CPU installation and cooling: Verify CPU installation, CPU power and CPU-cooler connection.

- Check GPU and display connection: Verify that the GPU is seated correctly, power connectors are attached, and the monitor cable is connected correctly.

- Check storage: Verify SATA power/data connections or M.2 installation and check whether the storage device appears in BIOS/UEFI.

- Check BIOS/UEFI: Confirm CPU, RAM and storage detection and check relevant configuration.

- Verify hardware compatibility: Confirm that the motherboard, CPU, RAM, storage, GPU and PSU are compatible.

- Test systematically: Disconnect unnecessary components and test the system using the minimum required hardware.

What I Changed and Why

The AI checklist was generally useful, but it did not make ESD precautions explicit before internal hardware handling. I added ESD and power-disconnection precautions near the beginning of the troubleshooting process. I also made the initial symptom and external power checks more explicit so that a technician does not immediately open the computer before checking simple causes. These changes make the checklist safer and more appropriate for an on-site junior technician.